Manual for the instruNet HP VEE drivers

These pages provide details of the compiled functions for programming the instruNet within HP VEE. This manual should be used in conjunction with the HP VEE manual, programming hints pages provided by Amplicon, and using the demos as examples. For further information on the calls, please refer to the Inet-int.c file provided with the instruNet software (view the file in Wordpad). In most cases the functions have the same name, and the parameters are the same.

# Copying the files into the right place

Before you can use the compiled functions for programming the instruNet within HP VEE, you must copy a couple of programs into the right place on the hard drive. To do this:

- Copy file iNet_Generic.h.txt to your HPVee Directory.

- Install instruNet hardware and software per the instruNet User's Manual (i.e. run the instruNet Setup.exe program, install hardware).

- Within HP VEE, Select PROGRAM EXPLORER under VIEW (in the menu bar) to hide it.

Using the Higher level function blocks:

It is worth noting, that this manual mostly applies to the lower level programming blocks. To make life even easier, the most popular of the functions are also presented, in easy to configure blocks in the example program “High_level_blocks.vee”. Both the high level blocks, and the low level blocks can be used in the same program by using the program above as your start program by following these steps:

Open HP VEE twice. Load High_level_blocks.vee, and your application in the other.

As you develop your application use copy (ctrl C) and paste (ctrl V) to copy the building blocks from High_level_blocks.vee over to your application.

The main difference with these high level blocks, is that there are generally no input terminals, because the settings can be selected in a settings window. To view this window, do a fast double click on the block to change the settings.

Using the lower level function blocks:

The first step in any program is to import the library of commands. This can be done by attaching a Start button (in the Flow menu) to an Import library function (Device menu), set to compiled functions specifying the location of the files, as described in the attached notes. Press the start icon to import the library.

The functions can now be placed in your program by one of three methods:

1) Select Call (in Device menu), then move cursor over box, do a right mouse click, and choose Select Function.

2) Press the Fx icon, then select Compiled Functions.

3) Move your cursor over the instruNet library block in one of the demo programs, and double click to expand it. Move around this until the desired function is found, then use copy and paste. It is not necessary to cut out the blocks connected to the input and output, as they are simply there as labels. Some of the functions in this block do not begin with “Call”. This means they have had some suggested values added, and have been converted into user objects. It is advisable that you use these suggested values.

Rather than looking at locations in RAM as specified by lines of code, HP VEE provides data on the black terminals of graphical blocks (data output is always the right hand side). Sometimes an input terminal on a Call Function block needs to see a block of memory, in those cases an array needs to go into the input terminal to simulate a section of RAM. For functions that output an array, a blank array needs to be connected to the input terminal. To create a blank array use an Allocate integer block (Data => Allocate Array => Integer). The Get Variable or Set Variable functions (in the Data => Variable submenu) can be used to simulate a single address in the same way (e.g., with PtrtoArg terminals). NB make sure a different variable name is used for different named terminals, also make sure that all variables are set to a value, usually zero, at the beginning of the program.

Descriptions of common parameters on input terminals (indicated by * in table)

NetNum: NETWORK number: {0...numNetworks}, (One iNET200 presents in each Network)

DeviceNum: DEVICE number: {0...numDevices},  0 = "Controller", 1 = 1st device on network etc

ModuleNum: MODULE number within a hardware device: {1...32} Most devices have only 1 module.

ChanNum: CHANNEL Number: {1...32}, Each device contains 32 channels in all, analog or digital, inputs or outputs, each of which has it's own channel number. The channel numbers can be clearly seen in instruNet world.

### MISCELLANEOUS USEFUL FUNCTIONS

| Function name | Description | Parameters | Possible values | Outputs |
| --- | --- | --- | --- | --- |
| Load_instrNet Driver It is advisable to just use the standard Import Library command | This imports the commands that will be used within HP VEE to control the instruNet. | Locations of DLL and .h files for compiled functions. | C:\Windows\system\iNet32.dll C:\program files\Hewlett-Packard\VEE 4.0 | RetValue |
| Open_instruNet_Window Easy | (Modified version of Open_instruNet_Window_Generic) This allows the user to set up the parameters of the test using specific windows within the instruNet World program rather than specific function calls. | Window type NetNum* DeviceNum* ModuleNum* ChanNum* SettingGpP FieldNumGP User PressedOkP | {1…15}Record/Network etc. page {0...numNetworks} {0...numDevices} {1...32} {1...32} {-13} advised {1} advised {1} advised | RetValue |
| Show_ALERT_if_hit_iNet_Error Generic | Checks to see if there was an error, and gives an error message if there was. | e | Value from any RetValue pin | RetValue |
| Get_instruNet_Network_State Generic | Turns all the settings of the instruNet into a 10Kbyte array, which can be saved using a standard “to file” function as a .prf file. For importing later using set_instruNet_Network_State | ArrayStart ArraySizeBytes numBytesOfDataP | Blank 10,000pt,1D array (see notes) {10000} {0} advised |  |
| Set_instruNet_Network_State Generic | Useful after a standard “from file” function where the file is a .prf configuration file. This will retrieve 10Kbyte of data of all the instruNet parameters as defined by a .prf file. | Arraystart dataSize | Output of a “from file” or similar array {10000} | RetValue Array |

## SLOW ANALOG AND DIGITAL I/O

| Function name | Description | Parameters | Possible values | Outputs |
| --- | --- | --- | --- | --- |
| GetChannelValue Generic | Returns the value of a specified channel on a specified device. This is returned in engineering units, or if digital channels, a value from 0 to 255. | NetNum* DeviceNum* ModuleNum* ChanNum* | {0...numNetworks} {0...numDevices} {1...32} {1...32} | Value RetValue |
| SetChannelValue Generic | Sets a voltage level on one of the DACs on a device or a digital output channel using an 8-bit word (0-255). NB: Not all of the 32 channels can be outputs. | NetNum* DeviceNum* ModuleNum* ChanNum* NewValue | {0...numNetworks} {0...numDevices} {1...32} {1…32} {0…255} | RetValue |

## FAST ANALOG AND DIGITAL I/O

| Function name | Description | Parameters | Possible values | Outputs |
| --- | --- | --- | --- | --- |
| AddToChannelList DB_Generic | (Modified version of AddToChannelList to remove Memory pointer). Place enabled channels in the digitize list. Each use of this command creates a new step in the channel list. These steps are {0…n}(see also GetDigitizedSegment which retrieves the samples) | NetNum* DeviceNum* ModuleNum* ChanNum* BufferSizeBytes * | {0...numNetworks} {0...numDevices} {1...32} {1...32} Use Get Variable function instead | RetValue |
| ResetChannelList Generic | Resets the list of channels in the channel list, previously set by AddToChannelList. List length becomes 0. | - | - | RetValue |
| Set_iNet_TIMING_Parameters Generic | Sets up the instruNet for timed sampling direct to RAM. All networks are set the same. If saving to disk, location can be set in “instruNet World” and saved in .prf file, which can be loaded later. Overflow means RAM buffer, as defined earlier, is full. | SamplesPerSecond PointsPerScan NumberOfScans StripChartRecorder SaveDataOption ShowAlertOnOverflow SwitchAccurately | {1…166K/No.Channels} {1…no. points} {1…no. of scans} {0= block scan, 1=continuous} {0=off, 2=to RAM, 3=to disk} {0=off, 1=on) {0=no (fast) 1=on (slow & acc.) | RetValue |
| Set_iNet_TRIGGER_Parameters Generic | This sets up whether there is hardware triggering or not. If off, then the other parameters are not important. | TriggerMode TriggerThreshold TriggerSlope PretriggerSeconds NetNum* DeviceNum* ModuleNum* ChanNum* | {0=off, 1=Auto, 2=Norm} {Ch. Value in engineering units} {0=rising, 1=falling} {0…seconds} {0...numNetworks} {0...numDevices} {1...32} {1...32} | RetValue |
| DigitzeListofChannels Generic | This is a software start. If trigger mode is off then sampling begins immediately, if trigger mode is on, then the instruNet waits for the trigger conditions to be met. | AllowMouseToStop… ProcessOneScanAtATime last_scanNum_base1… | {0=no, 1=mouse-down stops process} {0=no,1=get samples when complete} Number of last scan (start at 0) | RetValue Last_scan (incremented) |
| Stop_iNet_digitizing | Stops the instruNet whilst it is digitizing | - | - | RetValue |
| GetDigitizedSegment Vee Use copied user object called “GetDigitizeSegment” as provided in demo programs | It is advised that this function, rather than being used on it’s own, is used as a user object called “GetDigitizeSegment” copied from the inetscan.vee or the oscilloscope.vee demo program. It extracts sample from RAM for a specified step in the channel list. | Channel Number (terminal of user object) | {0...last step in channel list} | Array |

## ADVANCED FUNCTIONS FOR MODIFYING INDIVIDUAL VALUES IN THE INSTRUNET NETWORK

| Function name | Description | Parameters | Possible values | Outputs |
| --- | --- | --- | --- | --- |
| iNet_Generic | This is the main entry point for the instruNet DRIVER, that is exported out to a Win95 DLL The modifications here will appear in the “Settings” part of instruNet World See description in box below as well. | NetNum* DeviceNum* ModuleNum* ChanNum* SettingGroupNumOrType iNetINT16 fieldNum intention argType ptrToArg | {0...numNetworks} {0...numDevices} {1...32} {1...32} {0 or 1...# item in settingGroup menu} {1..Field number within the settingGroup} {intention_getValue, intention_setValue} {0…11} 0=int16…11=str255 Use Get variable instead of an address | RetValue PtrtoArg |
| iNet_rw_int16 / iNet_rw_int32 / iNet_cStr / iNet_rw_Double | Calls iNet() to read or write a field in the instruNet network with a signed 16bit/32bit/cStr value. To write to a field in the network, set the 'intention' input terminal to 'intention_setValue' and pass the new value into the 'iNet**Value' (** indicates where int16 etc to be). If you want to read the value of a field, set the 'intention' field to 'intention_getValue' and then refer to the returned value. | NetNum* DeviceNum* ModuleNum* ChanNum* SettingGroupNumOrType INet** fieldNum Intention PtrToArg | {0...numNetworks} {0...numDevices} {1...32} {1...32} {0 or 1...# item in settingGroup menu} {1..Field number within the settingGroup} {intention_getValue, intention_setValue} Use Get variable instead of an address | RetValue PtrtoArg |
| SetStaticVariable & GetStaticVariable | Sets or returns value of internal static variable. Used to store up to 10 static variables . initialized to 0. | Address Value | Use Get variable instead of an address |  |
| DoubleToFlt32_iNet_Vee & FltToDouble_iNet_Vee | Convert array of floating 32 bit int’s to an array of double or Visa versa. Source Addr and destination Addr can be the same buffer.(i.e. this will work with data in place) | SrcAddr_Flt32Array DestAddr_doubleArray NumPoints | Use Get variable instead of an address Use Get variable instead of an address Use Get variable | DestAddr… Or SrcAddr… |